Project  BRIEF

3D Scanning Module — Quality Improvement

| Platform iOS (Swift / SwiftUI, iOS 17+) |  |
| --- | --- |
| Scope 3D scanning module only (rest of app is complete) | Output Format .glb (primary) / .usdz (optional) |

# 1. Project Context

The client has a fully built iOS app for 3D head and body scanning. The core app (UI, navigation, Supabase backend, upload logic) is complete and working. The only failing component is the 3D scanning and reconstruction module, which produces poor-quality output. Your job is to improve or replace this module only — do not touch the rest of the app.

# 2. Existing Tech Stack

The current scanning module was built by a previous developer using the following:

| Component | Technology Used |
| --- | --- |
| UI Framework | Swift + SwiftUI (iOS 17+) |
| 3D Reconstruction | RealityKit ObjectCapture (photogrammetry) |
| Depth / AR | ARKit — LiDAR (rear) + Face Mesh TrueDepth (front) |
| Face Segmentation | VNDetectFaceLandmarksRequest (neck guillotine cut) |
| Camera | AVFoundation |
| Rendering | Metal / custom point cloud shaders |
| Export | .glb / .usdz |
| Backend | Supabase (model upload) |

# 3. Known Problems to Fix

The following issues have been identified as the root causes of poor quality output:

| Issue | Description |
| --- | --- |
| ObjectCapture misuse | ObjectCapture is designed for static inanimate objects — not for live human scanning. Humans breathe and micro-move, breaking photogrammetry alignment and producing fragmented, noisy meshes. |
| Guillotine neck cut | The current head/body segmentation uses VNDetectFaceLandmarksRequest to make a hard geometric cut at the neck. This creates jagged, unnatural transitions and visible seam artifacts. |
| No post-processing | There is no hole filling, mesh smoothing, decimation, or any cleanup pipeline after reconstruction. The raw output is delivered directly to export. |
| Weak depth fusion | Depth frames are not being fused across multiple passes. Each frame is treated in isolation, leading to incomplete surface coverage and missing geometry in occluded areas. |

# 4. Required Fixes (Minimum Viable Scope)

Given the fixed $1,000 budget, the following targeted improvements are expected — this is not a full pipeline rebuild, but a focused quality upgrade:

| Fix | Expected Outcome |
| --- | --- |
| Improved neck segmentation | Replace the hard landmark cut with a smoother boundary approach. Use ARKit body tracking or a landmark-weighted soft mask to produce a clean, natural neck transition with no visible seam. |
| Basic mesh post-processing | Add a cleanup pass after reconstruction: Laplacian smoothing to reduce noise, basic hole-filling for missing geometry, and triangle decimation to produce a clean, optimised mesh. |
| LiDAR capture tuning | Tune depth confidence thresholds to filter low-confidence depth readings. Improve multi-frame accumulation so depth data is aggregated across the scan session rather than single-frame only. |
| .glb export integrity | Ensure the exported .glb file is well-formed, complete, and compatible with standard 3D viewers. Texture mapping should align correctly with the mesh geometry. |

Note: A full pipeline replacement (TSDF volumetric fusion, Poisson reconstruction, AI refinement) is out of scope at this budget. The fixes above are the highest-impact improvements achievable within the constraint. Set this expectation with the client before starting.

# 5. What You Must NOT Change

The following parts of the app are complete and must remain untouched:

- App navigation and UI screens (SwiftUI)

- Supabase integration and model upload logic

- Authentication and user session handling

- Any existing .glb / .usdz downstream processing

- App Store metadata, entitlements, or project configuration

Deliver only the modified scanning module files. Provide clear documentation of every file changed.

# 6. Deliverables

- Modified Swift source files for the 3D scanning module

- Before/after scan comparison (at minimum 2 test scans on a human subject)

- Brief written summary of changes made and why

- Clean .glb output from a test scan demonstrating the quality improvement

# 7. Device Requirements

The app targets iPhones with LiDAR (iPhone 12 Pro and newer). Solutions must work on-device — no server-side reconstruction is in scope for this budget. Test on a physical LiDAR device, not the simulator.